GENERAL

SUMMARY

Section includes:

Summary of sustainable building requirements for the project to comply with the University of Washington Green Building Standard v2.

Summary of sustainable building requirements for the project as necessary for certification of this project through the US Green Building Council’s (USGBC) Leadership in Energy and Environmental Design version 4 for Building Design and Construction: New Construction Rating System (LEED NC v4). Commercial Interiors: Interior Design and Construction (LEED ID+C v4).

Related performance and administrative requirements.

- ______________________________________________________________________________

- Specifier – Consult with the PDG PM, GBS/LEED PM, or UW LEED Admin to determine the applicable LEED rating system (LEED NC or LEED ID+C):

- For LEED ID+C projects, delete all red highlighted text.

- For LEED NC projects, delete all magenta highlighted text.

- Consult with the PDG PM to determine the applicable Facilities Design Standard the project is vested under and update in 1.2B.

- Yellow highlighted text are LEED strategies to be customized for the project throughout design.

- Delete these instructions when the above tasks are completed.

- ___________________________________________________________________________

Related sections:

General Conditions and other Division 1 Specification Sections apply to this Section.

Related Sections include the following:

        - Section 01 25 00 	Substitution Procedures

        - Section 01 31 00 	Project Management and Coordination

        - Section 01 31 19		Project Meetings

        - Section 01 33 00 	Submittal Procedures

        - Section 01 35 15		Green Building Standard Requirements

        - Section 01 50 00		Temporary Facilities and Controls

        - Section 01 74 00		Construction & Demolition Waste Management

        - Section 01 91 00		General Commissioning Requirements

        - Section 01 91 19		Building Exterior Enclosure Commissioning

        - Sustainable criteria as noted in Division 2 through 48, including but not limited to other Sections listed in Section 01 35 15.

REFERENCES

University of Washington Green Building Standard v2 (UW GBS) available to download here: University of Washington Green Building Standard (uw.edu).

University of Washington Facilities Design Standards, [specifier – add MONTH/YEAR]

LEED-Building Design & Construction, New Construction (LEED NC), Interior Design and Construction (LEED ID+C) v4/v4.1 Rating System. Available as a free download from USGBC’s website www.usgbc.org .

LEED v4 Reference Guide for Building Design and Construction (LEED v4 BD+C New Construction Interior Design and Construction (LEED ID+C) Reference Guide). An essential supplement to the LEED NCv4/v4.1 Rating System.

A full discussion of credit and documentation requirements, green building issues, design approaches, calculation methodologies, references, definitions, case studies.

Available for a fee from USGBC web site.

LEED BD+C version 4.1 Rating System, Reference Guide, and other resources specific to the v4.1 beta are available for free from the USGBC web site at https://new.usgbc.org/leed-v41.

LEED Credit Forms: LEED On-Line with calculators and templates is available to team members at LEED-Online, https://www.usgbc.org/leedonline/.

Sheet Metal and Air Conditioning National Contractors Association (SMACNA) IAQ Guideline for Occupied Buildings Under Construction, 2nd Edition 2007, ANSI/SMACNA 008-2008 (Chapter 3).

ASHRAE 52.2-2007/2017 Air Filtration Standard

California Department of Public Health (CDPH) Standard Method v1.2-2017:

Testing of VOCs using Small Scale Environmental Chambers-v1.2-2017T

DEFINITIONS

Bio-based Materials: Bio-based materials meet the Sustainable Agriculture Network’s Sustainable Agriculture Standard. Excludes hide products, such as leather and other animal skin material.

Building exterior: A structure’s primary and secondary weatherproofing system, including waterproofing membranes and air- and water-resistant barrier materials, and all building elements outside that system.

Building interior: Everything inside a structure’s weatherproofing membrane.

Ceilings (product category): Includes all ceiling panels, ceiling tile, surface ceiling structures, such as gypsum or plaster, suspended systems (including canopies and clouds), and glazed skylights. Overhead structural elements (exposed, finished, and unfinished) are excluded.

Chain of Custody (CoC): A procedure that tracks a product from the point of harvest or extraction to its end use, including all successive stages of processing, transformation, manufacturing, and distribution.

Composite Wood (product category): Includes all particleboard, medium density fiberboard, hardwood veneer plywood, and structural Composite Wood not included in the flooring, Ceiling, Wall panels, or Furniture material categories.

Enclosure: The exterior plus semi-exterior portions of the building. Exterior consists of the elements of a building that separate conditioned spaces from the outside (i.e., the wall assembly). Semi-exterior consists of the elements of a building that separate conditioned space from unconditioned space or that encloses semi-heated space through which thermal energy may be transferred to or from the exterior or conditioned or unconditioned spaces (e.g., attic, crawl space, basement).

Environmental Product Declaration: Independently verified report based on peer-reviewed life-cycle assessment (LCA) studies that have been conducted according to a set of common product category rules (PCRs) for each product category.

Extended Producer Responsibility: Extended producer responsibility (also known as a closed-loop recycling program and as product take-back) puts a used product back into the production stream. Programs can be sponsored by the product’s manufacturer or other service.

Flooring (product category): Includes all types of hard and soft surface flooring (carpet, ceramic, vinyl, rubber, engineered, solid wood, laminates), wall base, underlayments, and other floor coverings.  Subflooring is excluded.

Furniture and Furnishings: Stand-alone Furniture items purchased for the project, including individual and group seating; open-plan and private-office workstations; desks and tables; storage units, credenzas, bookshelves, filing cabinets, and other case goods; wall-mounted visual-display products (e.g., marker boards and tack boards, excluding electronic displays); and miscellaneous items, such as easels, mobile carts, freestanding screens, installed fabrics, and movable partitions. Hospitality Furniture is included as applicable to the project. Office accessories, such as desktop blotters, trays, tape dispensers, waste baskets, and all electrical items, such as lighting and small appliances, are excluded.

GBS/LEED Project Manager: An individual responsible for the management of the UW GBS requirements and LEED certification.

General Emissions Evaluation: Testing of a product’s VOC content to determine compliance with the California Department of Public Health (CDPH) Standard Method.

Insulation (product category): Includes all thermal and acoustic boards, batts, rolls, and blankets, sound attention fire blankets, foamed-in place, loose-fill, blown, and sprayed Insulation. Insulation for HVAC ducts and plumbing piping are excluded.

LEED (Leadership in Energy and Environmental Design): Green building rating system developed by the US Green Building Council. The system certifies four levels (Certified, Silver, Gold and Platinum) of environmental achievement based on a point and credit scoring system

Local Materials: Materials that are extracted AND manufactured AND purchased from within 100 miles of Project location. Manufacturing refers to final assembly of components into building product that is installed at Project site. For composite products where a portion of total materials are regional; track the regional percentage.

LEED Credit Form to be completed within LEED Online, as specified by this Section.

Material Ingredient Reporting: A manufacturer’s report demonstrating the chemical inventory of a product or material to at least 0.1% (1000 ppm), using an approved program.

Materials Reuse: Reuse includes salvaged, refurbished, or reused products.

Post-Consumer Recycled Content: Percentage of waste material by weight available from consumer waste that has been used as a raw material (feedstock) in a building material.

Pre-Consumer Recycled Content: Percentage of waste material by weight available from industrial use incorporated into a building material.

Product and Material Cost: Includes all permanently installed building products.

      - MEP, specialty equipment, and items purchased for temporary use on the project are excluded.

Furniture and furnishings are not required to be included; however, they must be included if they are also included in MR BPDO EPDs and MR BPDO Material Ingredients.

Includes all taxes and expenses incurred by the contractor to deliver the material to the project site, but excludes cost for labor and equipment required for installation after the material is delivered to the site.

Equals cost per item times number of items purchased.

The cost of salvaged, reused or reclaimed materials is either the actual cost paid or the replacement value, whichever is higher.

Sustainable Agriculture Network Standard: Certification program for sustainable agriculture practices and products, sanstandards.org.

Sustainable Wood: Wood materials that are certified under the Forest Stewardship Council and have a CoC that tracks each product from source or location of harvesting to the distributor. “Wood” also includes bamboo and palm (monocots) as well as hardwoods (angiosperms) and softwoods (gymnosperms).

Volatile Organic Compounds (VOC): Organic compounds that easily become vapors or gases. Along with carbon, they contain elements such as hydrogen, oxygen, fluorine, chlorine, bromine, sulfur or nitrogen.

      - For Paints, VOC content does not include water or tint added at the point of sale.

UW LEED Admin: Individual responsible for UW-specific LEED assistance.

Wall Panels (product category): Includes all finish wall treatments (wall coverings, wall paneling, wall tile), surface wall structures such as gypsum or plaster, cubicle/curtain/partition walls, trim, doors, frames, windows, and window treatments. Removable/interchangeable fabric panels, built-in cabinetry, and vertical structural elements are excluded.

Definitions of other terms related to individual LEED credits: Refer to LEED v4 BD+C ID+C Reference Guide.

SUSTAINABLE BUILDING REQUIREMENTS

Integrate sustainable building materials and methods into Work as required to meet or exceed LEED v4 Gold Level certification from US Green Building Council. See LEED v4 Gold scorecard (updated at time of publication) in Figure 1: LEED Checklist.

Satisfy LEED prerequisites and credits applicable to Work of this Contract

Submit documentation that demonstrates this performance.

Participate as necessary to complete LEED certification application.

Conform to documentation requirements for LEED certification.

Adhere to the LEED Minimum Program Requirements (MPRs), including MPR #1 – Must Comply with Environmental Laws:

The LEED project building or space, all other real property within the LEED project boundary, and all project work must comply with applicable federal, state, and local building-related environmental laws and regulations in place where the project is located.

Refer to Contract Documents for incorporated sustainable/environmental requirements provisions.

Substitutions: Conform to provisions of Section 01 25 00 – Substitution Procedures.

DESIGN-BUILDER REQUIREMENTS

Coordinate and assist with documentation for LEED certification. Complete all documentation assigned to the Design-Builder.

Submit LEED Credit Forms and related documents directly to the GBCI via LEED-Online for review by GBS/LEED PM and/or UW LEED Admin.  Notify GBS/LEED PM and/or UW LEED Admin when documentation is ready for review.

GBS/LEED PM and/or UW LEED Admin to review documentation within 30 days of notification.

Respond to GBS/LEED PM and/or UW LEED Admin comments within 15 days.

Coordinate with GBS/LEED PM and/or UW LEED Admin to respond to GBCI review comments.

Submit Construction-Phase documentation (not including Commissioning prerequisite or credit) LEED Credit Forms and related documents directly to the GBCI via LEED-Online for review by GBS/LEED PM and/or UW LEED Admin within 90 days of substantial completion.

Designate a LEED Green Associate or LEED Accredited Professional. Representative responsible for:

Implementation, coordination, and documentation of specified LEED requirements.

Delivering the LEED Training Program

Reviewing LEED submittals to confirm complete before submitting for review

Scheduling and reporting at LEED meetings and providing meeting notes.

Maintain an electronic or hard copy of LEED v4 Reference Guide at project site construction office or in project digital files that are readily accessible on-site.

Ensure that all Subcontractors comply with the Project requirements for LEED certification including, but not limited to, documentation and reporting required by forms found in this Section.

LEED Training Program: The Design builder shall establish, manage, and implement a written Project specific LEED-procedures training program, and provide all necessary and appropriate instruction for its employees, and its Subcontractors of any tier, prior to their performance of Work at the Project Site in order to ensure the LEED requirements of the Project are met.

The LEED training program shall be submitted to the Sustainability Consultant for review and approval prior to the training being conducted.

Provide updated as necessary (upon new subcontract awards, etc.)

LEED Meetings:

Coordinate and attend LEED Pre-Construction Meeting to review LEED credits included in the project, methods, products and materials, tracking, and submittals for the project.

Arrange and conduct LEED Certification review meetings at regular intervals during construction coordinated with the PDG PM, GBS/LEED PM, and UW LEED Admin.

Each meeting will include a complete update of the status of LEED credits using relevant calculators or reporting tools which will also be published in the meeting notes.

______________________________________________________________________________________

- Specifier – Consult with the PDG PM, GBS/LEED PM, or UW LEED Admin to determine the applicable LEED credits the project is pursuing. Delete required submittals for credits the project is not pursuing.

- Green highlighted text is for healthcare projects only, delete if not applicable.

- Delete these instructions when the LEED credit strategy is finalized and the above tasks are complete.

_______________________________________________________________________________________

SUBMITTALS

Submit following in conformance to Section 01 33 00 – Submittal Procedures.

Include relevant LEED Materials Information Submittal Forms (Figure 2 below) with all materials submittals.

Include relevant LEED Materials Submittal Forms, provided at the end of this section, with all submittals.

Qualification Data: For Contractor's LEED Representative. Acceptable Qualification Data includes:

LEED AP Credential OR

LEED Green Associate Credential OR

Experience on (2) LEED projects

Letter of commitment from the contractor of capacity for 500 hours of LEED work

Progress Report Submittals:

Submit monthly to LEED PM (if different from contractor LEED Rep), UW LEED Admin, and UW PDG PM.

Confirm compliance for LEED prerequisites and credits by submitting a progress report that includes the following:

SS Prerequisite: Erosion and Sediment Control Photos -  Date-stamped photos of in-place erosion and sediment control measures taken, including any corrective action. Photos should be taken at regular intervals and show all areas of the site covered in the ESC Plan.

IAQ Credit: IAQ Photos - Beginning at project dry-in, provide date-stamped photos labeled with location and strategy highlighting the implemented construction IAQ practices demonstrating the SMACNA measures.

MR Credit: Construction and Demolition Waste Management – Current overall diversion rate for the Project

MR Credit: EPD Option 1/Option 2 – Current number of contributing products.

MR Credit: Sourcing of Raw Materials – Current progress towards 15/30% target. Must include actual total materials cost, cost for materials purchased to date and correlating sustainability attributes for each product.

MR Credit: Material Ingredient Reporting Option 1/Option 2 – Current number of contributing products.

IEQ Credit: Low-Emitting Materials – Current percent compliance for each material category the Project is pursuing.

IEQ Credit: Indoor Air Quality Assessment – Current plan and schedule for building flushout and/or IAQ Testing.

ID Credit: Social Equity within the Project Team – Current progress towards target metrics.

ID Credit: Procurement of Low Carbon Construction Materials – Current progress towards target metrics.

ID Credit: PBT Source Reduction – Lead, Cadmium and Copper – Current progress towards target metrics.

SS Prerequisite: Construction Activity Pollution Prevention.

Date-stamped photos of in-place erosion and sediment control measures taken, including any corrective action. Photos should be taken at regular intervals and show all areas of the site covered in the ESC Plan.

A summary, sample log, checklist, inspection report, or similar document that demonstrate periodic inspection of the implemented measures. This documentation must include:

Sample dates

Inspection frequency (at least monthly, year-round)

At least 3 inspections equally spaced over the site work period

Descriptions of any corrective action taken.

WE Prerequisite/Credit – Indoor Water Use Reduction

Provide product data for toilets, urinals, private lavatory faucets and showerheads indicating eligible products are WaterSense labeled.

Provide product data for flow fixtures (shower heads, lavatory faucets, pre-rinse spray valves and kitchen faucets) indicating flow rate in gallons per minute (gpm).

Provide product data for flush fixtures (water closets and urinals) indicating gallons per flush (gpf).

Provide product data for residential and commercial clothes washers, residential and commercial dishwashers and commercial ice machines indicating eligible products are Energy Star certified or meet the performance equivalent.

Provide product data for commercial dishwashers indicating water use in gallons per rack.

Provide product data for food steamers and combination ovens indicating water use in gallons per hour per pan.

Provide product data for food waste disposers, scrap collectors, and pulpers in gallons per minute (gpm).

Provide product data for strainer baskets confirming no additional water will be used by the equipment.

MR Prerequisite Construction & Demolition Waste (CDW) Management Planning

A CDW Management Plan that complies with Section 01 74 00.

Final report detailing all major waste streams generated, including disposal and diversion rates, diversion methods and facilities per Section 01 74 00.

Completed LEED Credit Form that includes a tabulation of total waste materials and quantity diverted, using consistent unit of measurement.

Support documentation:

Documentation of recovery rate (if co-mingled).

Waste hauling certificates or receipts.

MR Credit Construction & Demolition Waste Management (CDWM)

Completed LEED Credit Form including:

Option 1: Diversion

Completed CDWM Calculator

Total number of waste streams and diversion rate.

Narrative of diversion strategies prior to using waste-to-energy diversion.

Waste-to-energy documentation per Section 01 74 00

Option 2: Reduction of Total Waste Material/ Waste Prevention

Total new construction waste in pounds.

Diversion rate for renovation/demolition waste.

Confirmation that alternative daily cover is excluded from diverted waste but is included in total waste calculations.

MR Credits: Environmental Product Declarations (EPD), Sourcing of Raw Materials, and Material Ingredients:

Completed MR Calculator Materials Tab, including all products specified with contributing criteria, including Environmental Product Declarations (EPD); Sourcing of Raw Materials; and/or Material Ingredients. Include identification of manufacturer and material cost by line item.

Complete the LEED Materials Submittal Form (provided at the end of this Section) for all LEED submittals.

MR Credit: Environmental Product Declarations, Option 1

Completed MR Calculator EPD Tab

For each product provide a summary of the EPD, the full EPD document or a reference to the website where the publicly-available EPD can be accessed. Highlight relevant sections.

MR Credit: EPD, Option 2: Multi-Attribute Optimization

Completed MR Calculator EPD Tab, Option 1

For each product provide a summary of the Life-Cycle Impact Reduction document, the relevant sections of that document, or a reference to the website where the publicly-available document can be accessed. Highlight relevant sections.

For products meeting the criteria of this credit and meeting the definition of Regional Materials, also provide documentation showing product source location (extraction, manufacture, and purchase).

MR Credit: Sourcing of Raw Materials

Completed MR Calculator Sourcing of Raw Materials Tab

Complete the LEED Materials Submittal Form (provided at the end of this Section) for all LEED submittals.

Supporting documents (cutsheets, literature, etc.) demonstrating that products meet at least one of the responsible sourcing and extraction criteria: Extended Producer Responsibility, Bio-based Materials, Wood Products, Materials Reuse, Recycled Content.

For Extended Producer Responsibility: Documentation detailing takeback program, with proof that product purchased for project is included in the program.

For bio-based products: Documentation of compliance with either ASTM Test Method D6866, or the Sustainable Agriculture Network’s Sustainable Agriculture Standard.

For FSC wood products: Chain of custody documentation for FSC wood products.

For recycled content: Documentation showing percentages of post-consumer and/or post-industrial content, based on weight.

For products meeting at least one of the responsible sourcing and extraction criteria, and meeting the definition of Regional Materials, provide documentation showing product source location (extraction, manufacture, and purchase).

MR Credits: Material Ingredients Option 1: Material Ingredient Reporting

Completed MR Calculator Material Ingredients Tab, Option 1

Complete the LEED Materials Submittal Form for (provided at the end of this Section) for all LEED submittals.

Provide documentation of chemical inventory for all products contributing toward credit compliance. The inventory can be provided by the manufacturer, Health Product Declaration, Cradle to Cradle certification, Declare, UL Product Lens or another USGBC-approved program.

MR Credit: Material Ingredients, Option 2: Material Ingredient Optimization

Completed MR Calculator Material Ingredients Tab, Option 2.

Complete the LEED Materials Submittal Form for (provided at the end of this Section) for all LEED submittals.

Provide documentation demonstrating that products have optimized material ingredients through a Screening and Optimization Action Plan, Advanced Inventory & Assessment, or approved certification/label (manufacturer inventory, HPD, Cradle to Cradle certification, Declare, Cradle to Cradle v3, REACH, or other USGBC-approved program.

For products meeting the criteria of this credit and meeting the definition of Regional Materials, also provide documentation showing product source location (extraction, manufacture, and purchase).

IEQ Credit: Construction Indoor Air Quality Management Plan

Prior to beginning any work inside the building site, the Design-Builder shall develop and submit to the PDG PM, UW LEED Admin, and GBS/LEED PM for review a Construction Indoor Air Quality (IAQ) Management Plan.

Final IAQ Management Plan - The IAQ Management Plan shall include measures that comply with the following:

The five requirements of SMACNA IAQ Guideline for Occupied Buildings under Construction, 2nd Edition 2007, Chapter 3: HVAC protection, source control, pathway interruption, housekeeping, and scheduling and shall include:

List of IAQ protective measures to be instituted on the site:

HVAC system protection during construction.

Source control through specification and installation of low-toxic or non-toxic materials.

Pathway interruption to isolate work areas where emitting materials are being installed.

Housekeeping to protect materials that are stored before installation and to avoid spreading contamination through the Project.

Sequencing installation of materials to avoid contaminating absorptive materials during construction.

A no-smoking plan that prohibits the use of tobacco products inside the building(s) and within 25 feet (7.5 m) of the building entrances(s) during construction.

Identify procedures for protecting stored and installed absorptive materials from moisture damage.

Indicate whether air handlers will be operated during construction and include provision of appropriate filtration per this Section for any permanent equipment that will be used.

Post-construction indoor air quality assessment strategy to earn at least 1 point under the IAQ Assessment LEED credit for a building flush-out, or air quality testing. Projects may follow either version 4 or version 4.1 of this credit.

Schedule for inspection and maintenance of IAQ measures.

Completed LEED Credit Form for this credit declaring that credit requirements have been met.

Provide time- and date-stamped photos to highlight the implemented construction IAQ practices demonstrating the SMACNA measures.

If permanently installed air handling units were operated during construction, provide:

Location of AHUs used.

Provide cutsheets with manufacturer, model, and MERV rating of filters used at return air grilles during construction.

Pre-occupancy filter replacement date and MERV rating.

IEQ Credit: Indoor Air Quality Assessment

Completed LEED Credit Form declaring that credit requirements have been met.

Flushout or Air Testing Report.

Flushout report is to provide flush-out dates, temperature and humidity ranges, and monitoring protocol.

Air testing Report: provide an air testing report, highlighting the following:

Test dates

Concentration of formaldehyde, particulates (PM 10 or PM 2.5, as applicable), total volatile organic compounds (TVOC), target chemicals from CDPH Standard Method v1.2-2017, and carbon monoxide (CO), and ozone.

Concentrations of the contaminants listed in 3.12 AIR TESTING.

Test methods and any modifications made to the test methods. Explain how modifications, if any, assume that similar or better data are obtained.

IEQ Credit: Low-Emitting Materials

Completed LEED Credit Form declaring that credit requirements have been met.

Complete the LEED Low-Emitting Materials Information Submittal Form (included at the end of this Section) for all submittals.

Completed LEED Low Emitting Materials Calculator, which must include a list of adhesives, sealants, paints, coatings, flooring, composite wood, ceiling, wall and insulation, and furniture (if included in project scope) applied or installed onsite within the building’s weatherproofing system, with product name, manufacturer, applicable emissions evaluation, VOC content, and allowed VOC limit per LEED requirements.

For each of the products, submit cut sheets, MSDS, or letters from product manufacturers clearly indicating applicable emissions evaluation criteria used, applicable VOC regulation, TVOC levels or range, and VOC levels for each product.

Innovation Pilot Credit: Procurement of Low Carbon Construction Materials

Refer to LEED Pilot Credit Procurement of Low Carbon Construction Materials

Provide a third-party verified Environmental Product Declaration with applied UWCLF methodology and the total quantity of each material used in the construction of the project for the following materials:

Concrete (ready mixed concrete, slurry, shotcrete)

Masonry

Steel (rebar, steel wire and mesh, plate steel, structural steel, steel decking, cold formed steel, open-web steel joists)

Aluminum (aluminum extrusions, thermally improved aluminum extrusions)

Wood & Composites (dimensional lumber, plywood/OSB sheathing, glass mat gypsum sheathing, prefabricated wood products, composite lumber, mass timber)

Insulation (board, blanket, foamed-in-place, blown)

Finishes:

Gypsum Board

Acoustical Ceiling Tiles

Resilient Flooring

Carpet

Communications (data cabling)

Bulk Materials (flat glass)

Other materials as requested by the project team

_______________________________________________________________________________________

- Specifier – Consult with the PDG PM, GBS/LEED PM, or UW LEED Admin to determine the applicable LEED credits the project is pursuing. Delete product requirements for credits the project is not pursuing.

- Delete these instructions when the LEED strategy is finalized and the task is complete.

_______________________________________________________________________________________

PRODUCTS

LEED REQUIREMENTS

Comply with LEED requirements as noted in Technical Sections. Any Substitution requests must state whether or not proposed change impacts LEED requirements.

MR Credit: EPD, Option 1: Environmental Product Declaration (EPD)

Provide 20 10 products from at least five different manufacturers that meet one of these Life-Cycle Assessment and Environmental Product Declarations disclosure criteria:

Life-Cycle Assessments and Environmental Product Declarations (valued as 1 product):

Publicly available, critically reviewed LCA conforming to ISO 14044, and with at least a cradle to gate scope.

Product-specific Type III EPD -- Internally Reviewed. Products with an internally critically reviewed LCA in accordance with ISO 14071. Products with product-specific internal EPDs that conform to ISO 14025, and EN 15804 or ISO 21930, and have at least a cradle to gate scope.

Industry-wide Type III EPD -- Products with third-party certification (Type III), including external verification, in which the manufacturer is explicitly recognized as a participant by the program operator.  Products with industry-wide EPDs that conform to ISO 14025, and EN 15804 or ISO 21930, and have at least a cradle to gate scope.

EPDs that conform to ISO 14025, and EN 15804 or ISO 21930, and have at least a cradle to gate scope (valued as 1.5 products):

Product-specific Type III EPD -- Products with third-party certification (Type III), including external verification and external critical review in which the manufacturer is explicitly recognized as the participant by the program operator.

For products extracted, manufactured and purchased/distributed within 100 miles of the project site: Submit City/State/Country of material extraction, and distance from each location to project site.

Contractor is responsible for procuring adequate number of qualifying products for credit achievement.

MR Credit: EPD, Option 2: Multi-Attribute Optimization

Provide 5 permanently installed products from at least three different manufacturers that comply with one of the criteria below.

Life-Cycle Impact Reduction Action Plan:  Submit a copy or provide a link to the manufacturer’s action plan. A qualifying plan must be publicly available, critically reviewed, and based on a product-specific LCA using EN 15804 or ISO 21930 and the specified PCR functional unit. Must include the following information:

Description of the LCA conducted including the dataset, software or platform used to complete the analysis.

Identification of the largest life cycle impact areas and a narrative description of the impact areas targeted for reduction.

Description of specific impact reduction strategies, including proposed changes in formulation or manufacturing processes.

Specific dates and a full timeline for completion.

Life-Cycle Impact Reductions in Embodied Carbon:  Submit a copy or provide a link to the current third-party EPD or verified LCA, conforming to comparability requirements of ISO 14025 and ISO 21930. Comparative analysis must show a reduction in the global warming potential (GWP) impact category and include a narrative describing how reductions were achieved. Published comparisons must be third-party verified. Product count is based on the criteria:

Reduction in (GWP) is valued at 1 product.

At least 10% reduction in (GWP) is valued at 1.5 products.

At least 20% reduction in (GWP) and at least 5% reduction in two additional impact categories is valued at 2 products.

For products extracted, manufactured and purchased/distributed within 100 miles of the project site: Submit City/State/Country of material extraction, and distance from each location to project site.

Contractor is responsible for procuring adequate number of qualifying products for credit achievement.

MR Credit: Sourcing of Raw Materials: Responsible Sourcing of Raw Materials

Provide products sourced from at least five different manufacturers that meet one or more of the criteria below for at least 15%/30%, by cost, of the total value of permanently installed building products in the project:

Extended producer responsibility: Submit documentation detailing takeback program, with proof that product purchased for project is included in the program (valued at 50% of cost).

Bio-based materials: Submit documentation verifying bio-based product/material was tested using ASTM Test Method D6866 (valued at 50% of cost), or meets the Agriculture Network’s Sustainable Agriculture Standard (valued at 100% of cost).

FSC wood products: Submit chain of custody documentation on final invoice (valued at 100% of cost).

Materials reuse: Submit documentation indicating origin, age, and cost of salvaged, refurbished, or reused products (valued at 200% of cost).

Recycled content: Submit documentation indicating postconsumer and pre-consumer recycled content percentage and cost (valued at 100% of cost).

For products extracted, manufactured and purchased/distributed within 100 miles of the project site: Submit City/State/Country of material extraction, and distance from each location to project site.

Products proposed for substitution must meet original product LEED specification criteria to meet 15%/30% threshold.

Contractor is responsible for procuring adequate number of qualifying products for credit achievement.

MR Credits: Material Ingredients Option 1: Material Ingredient Reporting

Products in the Technical Sections have been specified to achieve this credit, using at least 20 10 different permanently installed products from at least five different manufacturers that use any of the following programs to demonstrate the chemical inventory of the product to at least 0.1% (1000 ppm): Manufacture Inventory; Health Product Declaration; Cradle to Cradle; Declare; UL Lens Certification; or USGBC approved program.

Submit any third-party verification that includes content inventory, for any of the programs listed (valued as 1.5 products).

For products extracted, manufactured and purchased/distributed within 100 miles of the project site: Submit City/State/Country of material extraction, and distance from each location to project site.

Contractor is responsible for procuring adequate number of qualifying products for credit achievement.

MR Credit: Material Ingredients, Option 2: Material Ingredient Optimization

Provide 5 compliant permanently installed products, from at least three different manufacturers that use one of the following approaches or programs to disclose optimized material ingredient information:

Material Ingredient Screening and Optimization Action Plan (valued at ½ product):  Submit a copy or a link to the manufacturer’s Screening and Optimization Action Plan, which includes a materials inventory meeting the requirements of Option 1. The plan must be product-specific (not company, manufacturer or brand), and include the following information:

Description of the screening or assessment platform used to complete the material ingredient screening and analysis.

Identification of the specific green chemistry principles targeted for implementation.

Description of specific implementation strategies, including proposed changes in formulation or manufacturing processes.

Specific dates and a full timeline for completion.

Advanced Inventory & Assessment: Submit verification of product compliance with any of the following programs:

Manufacturer Inventory

Health Product Declaration

Declare

Cradle to Cradle v3

Material Ingredient Optimization: Submit verification that product has an inventory and assessment of ingredients using one of the following programs:

Manufacturer Inventory

Health Product Declaration

Cradle to Cradle v3

For products extracted, manufactured and purchased/distributed within 100 miles of the project site: Submit City/State/Country of material extraction, and distance from each location to project site.

Contractor is responsible for procuring adequate number of qualifying products for credit achievement.

Innovation Pilot Credit: Procurement of Low Carbon Construction Materials

Provide lower embodied carbon products with third-party verified Environmental Product Declaration with applied UWCLF methodology used in the construction of the project for the following materials:

Concrete (ready mixed concrete, slurry, shotcrete)

Masonry

Steel (rebar, steel wire and mesh, plate steel, structural steel, steel decking, cold formed steel, open-web steel joists)

Aluminum (aluminum extrusions, thermally improved aluminum extrusions)

Wood & Composites (dimensional lumber, plywood/OSB sheathing, glass mat gypsum sheathing, prefabricated wood products, composite lumber, mass timber)

Insulation (board, blanket, foamed-in-place, blown)

Finishes:

Gypsum Board

Acoustical Ceiling Tiles

Resilient Flooring

Carpet

Communications (data cabling)

Bulk Materials (flat glass)

Other materials as requested by the project team

- Specifier –Consult with GBS/LEED PM to identify the project’s LEED Low Emitting Materials categories. Customize all product category references below to the project’s LEM strategy.

- Delete these instructions when the LEM strategy is finalized and the updates are complete.

- ______________________________________________________________________________

LOW EMITTING MATERIALS

Products applied onsite, installed within the Project’s waterproofing membrane and defined as Adhesives and Sealants, Paints and Coatings, Flooring, Composite Wood, Ceilings, Wall panels, Insulation, and/or Furniture. Must comply with the following requirements, as applicable:

Inherently non-emitting sources: Products that are inherently non-emitting sources of VOCs (stone, ceramic, powder-coated metals, plated or anodized metal, glass, concrete, clay brick, and unfinished or untreated solid wood flooring) are considered fully compliant without any VOC emissions testing if they do not include integral organic-based surface Coatings, binders, or Sealants.

General Emissions Evaluation: Adhesives and Sealants, Paints and Coatings, Flooring, Ceilings, Wall panels and/or Insulation must be tested and determined compliant with the California Department of Public Health (CDPH) Standard Method v1.2–2017 or later, using the applicable exposure scenario. The default scenario is the private office scenario.

Manufacturer or third-party certification must state exposure scenario used to determine compliance.

Claims of compliance for wet-applied products must state the amount applied in mass per surface area.

Manufacturers’ claims of compliance must also state the range of total VOCs after 14 days (336 hours), measured as specified in the CDPH Standard Method v1.2–2017:

0.5 mg/m3 or less;

between 0.5 and 5.0 mg/m3; or

5.0 mg/m3 or more.

VOC Content: Adhesives, Sealants, Paints and Coatings must meet applicable VOC content limits:

Paints and Coatings must meet the applicable VOC limits of the California Air Resources Board (CARB) 2007 Suggested Control Measure (SCM) for Architectural Coatings - https://ww2.arb.ca.gov/our-work/programs/coatings/architectural-coatings/voc-limits, or the applicable “Effective Date” VOC limits per South Coast Air Quality Management District (SCAQMD) Rule 1113, effective February 5, 2016 - https://www.aqmd.gov/home/rules-compliance/compliance/vocs/architectural-coatings/tos.

Adhesives and Sealants must meet SCAQMD Rule 1168, October 6, 2017, (Adhesives and Sealant Applications) - RULE 1168 ADHESIVE AND SEALANT APPLICATIONS (aqmd.gov) . Includes laminating adhesives applied on site.

If the applicable regulation requires subtraction of exempt compounds, any content of intentionally added exempt compounds larger than 1% weight by mass (total exempt compounds) must be disclosed.

Methylene chloride and perchloroethylene may not be intentionally added in Paints, Coatings, Adhesives, or Sealants.

Formaldehyde Emissions Evaluation: Structural and Non-structural composite wood products must meet the following formaldehyde emissions evaluation, by cost or surface area.

Non-structural Composite Wood products must meet one of the following:

EPA TSCA Title VI or California Air Resources Board (CARB) ATCM for formaldehyde requirements for ultra-low-emitting formaldehyde (ULEF) or no added formaldehyde (NAF) resins. See Table 1 Urea Formaldehyde Limits for Ultra-Low-Emitting Formaldehyde (ULEF) Resins.

Tested per EN 717-1:2014 for formaldehyde emissions and complies with emissions class E1.

Structural Composite Wood products must use moisture resistant Adhesives meeting ASTM D2559, have no surface treatments with added urea-formaldehyde resins or Coatings, and certification of compliance with industry standard for that product type.

Wood Structural Panel- manufactured according to PS 1-09 or PS 2-10 (or CARB equivalent) and labeled bond classification Exposure 1 or Exterior.

Oriented strand board: specified with the Exposure 1 or Exterior bond classification in accordance with Voluntary Product Standard – Performance Standard for Wood‐Based Structural‐Use Panels (PS 2‐10)

Structural Composite Lumber- manufactured according to ASTM D5456.

Glued Laminated Timer- manufactured according to ANSI A190.1.

I-Joists- manufactured according to ASTM D 5055.

Cross Laminated Timber- manufactured according to ANSI PRG 320.

Finger-jointed lumber- manufactured according to PS 20-15.

Furniture Emissions Evaluation: New Furniture and Furnishings must be tested in accordance with ANSI/BIFMA Standard Method M7.1-2011 (R2016); comply with ANSI/BIFMA e3-2014e or e3-2019eFurniture Sustainability Standard, Sections 7.6.1 and 7.6.2, using either the concentration modeling approach or the emissions factor approach; and model the test results using the open plan, private office, or seating scenario in ANSI/BIFMA M7.1, as appropriate. See Table 2 Furniture Emissions Limits.

The thresholds of compliance by product category are listed below:

Wet-applied products:

75%, by volume or surface area, of products defined as Adhesives and Sealants must meet the General Emissions Evaluation.

75%, by volume or surface area, of products defined as Paints and Coatings must meet the General Emissions Evaluation.

100% of products defined as Paints and Coatings must meet the applicable VOC limits.

100% of products defined as Adhesives and Sealants must meet the applicable VOC limits. Includes laminating adhesives.

Flooring: 90%, by cost or surface area, of products defined as Flooring must meet the General Emissions Evaluation.

Insulation: 75%, by cost or surface area, of products defined as Insulation must meet the General Emissions Evaluation.

Ceilings: 90%, by cost or surface area, of products defined as Ceilings must meet the General Emissions Evaluation.

Wall Panels: 75%, by cost or surface area, of products defined as Wall Panels must meet the General Emissions Evaluation.

Composite Wood: 75%, by cost or surface area, of products defined as Composite Wood must meet the applicable formaldehyde emissions evaluation.

Furniture and Furnishings: 75%, by cost, of products defined as new Furniture and Furnishings must meet the applicable furniture emissions evaluation.

Refer to USGBC Compliant Third Party Certifications and Labels for Low Emitting Materials for updates and most current program dates and documents: http://www.usgbc.org/resources/low-emitting-materials-third-party-certification-table

| - Table 1 Urea Formaldehyde Limits for Ultra-Low-Emitting Formaldehyde (ULEF) Resins |  |  |  |
| --- | --- | --- | --- |
|  | Particleboard | MDF | Thin MDF |
| ULEF-target | 0.05 ppm | 0.06 ppm | 0.08 ppm |
| ULEF-cap | 0.08 ppm | 0.09 ppm | 0.11 ppm |

| Table 2 - Furniture Emissions Limits |  |  |
| --- | --- | --- |
| Workstation systems (open plan or private) office emissions concentration limits |  |  |
| TVOC toluene | ≤ 0.5 mg/m3 |  |
| Formaldehyde | ≤ 50 ppb |  |
| Total Aldehydes | ≤ 100 ppb |  |
| 4-Phenylcyclohexene | ≤ 0.0065 mg/m3 |  |
| Seating office emissions concentration limits |  |  |
| TVOC toluene | ≤ 0.25 mg/m3 |  |
| Formaldehyde | ≤ 25 ppb |  |
| Total Aldehydes | ≤ 50 ppb |  |
| 4-Phenylcyclohexene | ≤ 0.00325 mg/m3 |  |
| Individual Furniture components maximum emissions factors |  |  |
|  | ANSI/BIFMA M7.1 2011 Open Plan Workstation | ANSI/BIFMA M7.1 2011 Private Office Workstation |
| Formaldehyde (µg/m2 hr) | 42.3 | 85.1 |
| TVOC (µg/m2 hr) | 345 | 694 |
| Total Aldehyde (µmol/m2 hr) | 2.8 | 5.7 |
| 4-Phenylcyclohexene (µg/m2 hr) | 4.5 | 9.0 |

EXECUTION

LEED COMPLIANCE - GENERAL

Meet LEED Prerequisites and Credits as identified in the LEED Checklist in Figure 1 at the end of this Section, as required for achieving LEED Certification.

Prior to beginning Work of this Contract, verify construction conditions as acceptable to achieve LEED Credit and Prerequisite requirements.

Do not proceed with Work until unsatisfactory conditions are corrected in a manner acceptable to PDG PM, UW LEED Admin, GBS/LEED PM, and Architect.

Correct non-conforming work failing to meet LEED requirements at Contractor’s expense.

Submit documentation as necessary to show conformance of corrected work.

PDG PM, UW LEED Admin, GBS/LEED PM and USGBC will be sole judge in determining conformance to LEED Credit requirements.

At the end of project, UW LEED Admin to upload LEED Documentation Snapshot downloaded from LEED Online, final LEED Review Report, and OPR to the UW Sustainability G Drive.

COORDINATION

Coordinate with all subcontractors per Section 01 74 00.

Coordination shall include review of this Section 01 35 15.01 and related sections.

Include subcontractors in related meetings, including LEED meetings, per Section 01 31 19.

LEED APPLICATION

The Owner will pay all fees to register and certify the Project with GBCI.

The Owner will provide the Contractor access rights to the GBCI project web site.

EROSION & SEDIMENTATION CONTROL

SS Prerequisite: Conform to intent of this Prerequisite, Civil Drawings, and provisions of Division 31, including Section 31 10 00.

CONSTRUCTION AND DEMOLITION WASTE MANAGEMENT

MR Prerequisite Construction and Demolition Waste Management Planning and MR Credit Construction and Demolition Waste Management: Meet objectives and diversion rates established in the Construction and Demolition Waste Management Plan; see Section 01 74 00.

IAQ MANAGEMENT DURING CONSTRUCTION

IEQ Credit Construction IAQ Management:

Implement IAQ management plan before starting above-ground construction, storing materials on-site, or roughing in mechanical systems.

Photograph IAQ measures regularly and provide annotations of measures documented.

Use safety meetings, signage, and subcontractor agreements to communicate the goals of the construction indoor air quality management construction plan.

Post a copy of the IAQ management plan on site, preferably posted in an accessible area, and translated into the language(s) spoken by subcontractors and their crew.

Conduct regular inspection and maintenance of indoor air quality measures including ventilation system protection, and ventilation rate.

IEQ Credit IAQ Assessment:

Pre-occupancy flushout

After construction is complete, including installation of all interior finishes, movable furnishings provided by PDG PM, and major VOC punch list items, and prior to occupancy, perform flush-out of building:

Install new permanent filtration media prior to beginning of flush-out

Supply total air volume of 14,000 cu ft. of outdoor air per sq. ft. of floor area

Maintain an internal building temperature of at least 60 degrees F and no higher than 90 degrees F

Maintain an internal building relative humidity no higher than 60%

- OR

Flush out during occupancy

After construction is complete, including installation of all interior finishes, movable furnishings provided by PDG PM, and major VOC punch list items, and prior to occupancy, perform flush-out of building:

Prior to occupancy:

Supply total air volume of 3,500 cu ft. of outdoor air per sq. ft. of floor area

Maintain an internal building temperature of at least 60 degrees F and no higher than 90 degrees F

Maintain an internal building relative humidity no higher than 60%

During each day of occupancy:

Ventilation must begin at least three hours prior to occupancy and continue during occupancy, until a total of 14,000 cu ft. of outdoor air per sq. sf has been delivered to the space.

Outdoor air just be delivered at a minimum rate of 0.30 cfm of outdoor air per sq. ft. or at the design minimum outdoor air rate, whichever is greater.

OR

Air Testing

After construction is complete, including installation of all interior finishes, movable furnishings provided by PDG PM, and major VOC punch list items, and prior to occupancy, perform flush-out of building

Reference the appropriate Reference Guide for the version of the LEED credit selected and perform test to the reference standards listed within the credit. Document any modifications made to the test methods.

COMMISSIONING

EA Prerequisite and EA Credit for Enhanced Commissioning: Conform to Commissioning Plan as designated by Commissioning Authority. Implement contractor’s responsibilities for systems manual, training, and seasonal testing.

EA Credit for Enhanced Commissioning – Envelope Commissioning: Conform to Envelope Commissioning Plan as designated by Commissioning Authority. Implement contractor’s responsibilities for systems manual, training, and seasonal testing.

INNOVATION

TBD

Pilot Credit: Social Equity within the Project Team

Demonstrate commitment to equity for construction workers through workforce pay and benefits by achieving both of the following:

60% or more of the construction value performed by contractors paying worker wages and benefits that meet or exceed the prevailing wage determined by the Federal Davis-Bacon Act, or applicable State prevailing wage statutes

- AND

15% or more of total project construction hours performed by participants enrolled in Job-Related Skills training through on the job training in a Department of Labor registered apprenticeship program

- OR

Life-Skills training programs conducted on the construction site including such things as GED test preparation, English as second language (ESL) courses, financial literacy, debt management, first-time home buying, or entrepreneurship training. Must deliver at least one course per month for the duration of construction

- OR

60% or more of the construction value performed by contractors providing financial assistance in the form of scholarships, stipends, or sponsorships for workers to attend life-skills training programs conducted off-site such as pre-apprenticeship training, English as a second language (ESL) courses, GED test preparation, financial literacy, debt management, first-time home buying, or entrepreneurship training

- OR

USGBC-approved equivalent

Figure 1: LEED Scorecard (Specifier – request from LEED PM and then delete this highlighted instruction)

Figure 2: LEED Submittal Coversheet

GBS/LEED PM to provide digital version of preferred version of this form.

- END OF SECTION